Challenges for commercialization of solar cells:

Immense research is going on solar cells related to compositions, additives, deposition methods, a broad range of device architectures, and a wide variety of selective charge-extraction layers.

There are several challenges regarding perovskite solar cells to make them commercialize. Signiﬁcant eﬀorts have been dedicated to improving perovskite devices by controlling the perovskite precursor composition, processing parameters, and deposition techniques.

Compositions with ideal bandgap

- There is need to seek lower perovskite bandgaps, which are more suitable for single junction solar cells.

- Cation substitution or blending has also been employed for ﬁne-tuning the bandgap and enhance stability.

- Although thermal stabilities and device lifetimes are improved for all-inorganic perovskites compared to those for hybrid organic inorganic perovskite solar cells (HOIPs), the photoconversion efficiency observed for all-inorganic perovskites are generally lower than those for HOIPs-based SCs.

- Need to suppress the photoinactive yellow-phase impurities (hexagonal δ-phase) and the residual cubic PbI2 phase.

- Proper combined perovskite passivation with Lewis acids and bases minimizes the loss of the organic cation at elevated temperatures and reduces moisture ingress through grain boundaries, allowing for improved stabilities.

- Cs doping approach requires careful design to stabilize the desired cubic (α) phase and to avoid the thermodynamically favoured transformation into the undesired orthorhombic (δ) phase.

- Spiro-OMeTAD and PTAA are among the most widely used organic HTMs in PSCs, signiﬁcant eﬀorts have been devoted to replacing them, as they are rather expensive.

- Use of inexpensive inorganic hole extraction layers such as NiO, CuI, Cs2SnI6, and CuSCN.

- Developing new compositions that could oﬀer better stability in severe conditions.

- Examining lead-free options (e.g., based on germanium, tin, bismuth, or antimony), perhaps with the assistance of additives or through bimetallic structures, for improving their current stabilities.

Stability

- Some perovskites lack structural stability at room temperature due to possible crystallization into undesired phases, such as the photoinactive hexagonal δ-phase.

- Improvement in crystallinity for reducing hysteresis and photovoltage loss.

- HOIPs have inherent challenges pertaining to the desorption of their organic species (e.g., MA) during thermal annealing, resulting in the formation of vacancies.

- Utilized temperature and ligand length during NC synthesis play important roles in controlling the morphology of the perovskite NCs.

- Colloidal perovskite NCs are soft and chemically unstable substances; hence, it is necessary to establish methods for stabilizing the NCs against light, temperature, and the environment.

- Polymer coating, silica coating, and X-ray irradiation have evolved as other promising post synthetic treatments for improving the photostability of perovskite NCs. However, these strategies are expected to leave the NCs inaccessible in terms of injection of charges

- Stabilizing of colloidal perovskite NCs by overcoating them to produce core−shell structures.

Interface engineering

- Post-treatment strategy that improve electronic coupling between the nanocrystals.

- Perovskite materials engineering.

- Deposition of high-quality ﬁlms.

- strain engineering, and passivation, which improve perovskite stability against heat, light, and moisture exposure.

- Limited short-circuit current density (JSC) due to the presence of capping ligands that cause high electric resistance.

- Need to improve the electronic coupling between QDs, thus enhancing carrier mobility.

- In bilayer architecture, perovskite overlayer need much larger grain size (100−1000 nm), which is necessary for better charge transport and extracting charge carriers to attained high eﬃciencies with less hysteresis.

- Seeking the deposition of a more uniform and homogeneous ﬁlm with other deposition techniques.

- Proper Interfacial engineering has to be provided for an unparalleled pathway for suppressing recombination and carrier losses and subsequently improving the overall optoelectronic quality of perovskites.

- Interfacial band bending, kinetics of charge extraction and recombination at the device interface.

- The instability of perovskites and their soft, ionic chemical nature are mainly responsible for the diﬃculties in applying conventional epitaxial core−shell growth, which encourages testing other strategies, such as nonepitaxial shell deposition and room-temperature solution based atomic layer deposition.